PROBLEM STATEMENT :

A smart manufacturing company plans to develop an energy-efficient processor for an industrial  monitoring system. The processor should efficiently handle sensor data and real-time processing requirements. Analyze the functional units, including CPU, memory, buses, and I/O components. Apply the instruction execution cycle and addressing modes for efficient instruction processing. Evaluate processor performance using CPI, clock cycles, execution time, and benchmarking. Design a suitable pipelined datapath to improve instruction throughput. Analyze data, control, and

structural hazards occurring during pipeline execution. Apply suitable techniques such as forwarding,  stalling, and branch prediction to reduce hazards. Compare superscalar processing, out-of-order execution,  and speculative execution. Analyze the suitability of simultaneous multithreading (SMT) for improving  processor utilization. Design a hierarchical memory system using L1, L2, and L3 caches with virtual memory.

Compare SRAM, DRAM, NAND Flash, MRAM, and RRAM based on latency, bandwidth, throughput, and  power. Recommend an optimized processor and memory organization for improved performance and energy  efficiency.

Justify the proposed design based on performance, power consumption, and resource utilization. Explain how the design supports SDG 7, SDG 9, and SDG 12 through clean energy use, innovation, and  responsible resource consumption.

# 1. INTRODUCTION

Modern smart manufacturing systems continuously collect data from sensors such as temperature, pressure, vibration, humidity and machine-speed sensors. The processor used in such a system must process sensor information in real time while consuming minimum power.

The proposed system is therefore designed as an energy-efficient industrial monitoring processor. It combines:

- CPU and ALU

- Register file

- Control unit

- Pipelined datapath

- L1, L2 and L3 cache

- Virtual memory

- Memory controller

- System bus

- Sensor and I/O interfaces

- Low-power execution techniques

The architecture is designed to provide a balance between performance, latency, power consumption, cost and reliability.

# ASSESSMENT 1

# FUNCTIONAL UNITS, INSTRUCTION EXECUTION, ADDRESSING MODES & PERFORMANCE ANALYSIS

## CO1:

## 1.1 Proposed Processor Architecture

The proposed processor contains the following major functional units.

| Functional Unit | Function | Application in Industrial Monitoring |
| --- | --- | --- |
| Control Unit | Controls instruction execution | Controls sensor-data processing |
| ALU | Performs arithmetic and logical operations | Temperature, pressure, and vibration calculations |
| Register File | Stores temporary operands/results | Fast storage for sensor values |
| Program Counter (PC) | Stores address of next instruction | Controls instruction sequence |
| Instruction Register | Stores current instruction | Holds instruction during decoding |
| Cache Memory | Stores frequently used data/instructions | Reduces memory access latency |
| Main Memory | Stores programs and data | Stores monitoring software and sensor data |
| Memory Controller | Controls memory transactions | Coordinates CPU and DRAM |
| Bus | Transfers data, addresses, and control signals | Connects CPU, memory, and I/O |
| I/O Controller | Handles external devices | Interfaces with sensors and actuators |
| Interrupt Controller | Handles urgent events | Responds quickly to abnormal sensor values |

# 1.2 System Architecture

### Proposed architecture

Industrial Sensors

Temp / Pressure etc.

.

I/O Unit

ALU

Arithmetic & Logic Ops

CPU

Control Unit

Registers & Instructions

CPU

L3 Cache

L2 Cache

L1 Cache

Main Memory DRAM

Secondary Storage

# 1.3 CPU Functional Units

### Control Unit

The control unit interprets instructions and generates control signals.

Applied example:

When a temperature sensor sends a value, the CPU may execute:

LOAD → ADD → COMPARE → BRANCH → STORE

The control unit ensures that each instruction is executed in the correct sequence.

### ALU

The ALU performs:

- Addition

- Subtraction

- Comparison

- AND

- OR

- XOR

- Shift operations

For example:

Temperature = 85°C

Threshold   = 80°C

ALU:

85 > 80

The comparison result can trigger an alarm.

### Registers

Registers provide extremely fast temporary storage.

Example:

R1 = Temperature

R2 = Threshold

R3 = Result

Instruction:

CMP R1, R2

The result is then used to determine whether the machine is operating safely.

# 1.5 Addressing Modes

Different addressing modes can be applied according to the sensor-processing requirement.

| Addressing Mode | Example | Application |
| --- | --- | --- |
| Immediate | ADD R1, #10 | Add fixed threshold |
| Register | ADD R1, R2 | Sensor calculations |
| Direct | LOAD R1, 1000 | Access fixed memory location |
| Indirect | LOAD R1, (R2) | Access sensor buffer |
| Indexed | LOAD R1, 100(R2) | Array processing |
| Base + Offset | LOAD R1, 8(R2) | Structure access |

### Analysis

Immediate addressing is efficient when constants are used.

Register addressing is faster because it avoids memory access.

Indexed addressing is useful for sensor arrays.

For example:

Sensor[0]

Sensor[1]

Sensor[2]

Sensor[3]

An indexed address can efficiently access each sample.

# 1.6 Performance Analysis

The main performance parameters are:

- CPI

- Clock cycles

- Clock frequency

- Execution time

- Throughput

- Benchmark performance

### Formula

## Example Calculation

Assume:

- Instruction count = 1,000,000

- CPI = 1.5

- Clock frequency = 1 GHz

### Clock cycles

### Execution time

### L4 Analysis

Reducing CPI directly reduces execution time if instruction count and clock frequency remain constant.

For example, if CPI is reduced from 1.5 to 1.2:

Therefore, the execution time improves from 1.5 ms to 1.2 ms.

RESULT

| Parameter | Value |
| --- | --- |
| Instruction Count | 1,000,000 |
| Original CPI | 1.5 |
| Improved CPI | 1.2 |
| Clock Frequency | 1 GHz |
| Original Execution Time | 1.5 ms |
| Improved Execution Time | 1.2 ms |
| Improvement | 20% |

# ASSESSMENT 2

# PIPELINED PROCESSOR ORGANIZATION & HAZARD ANALYSIS

## CO3:

# 2.1 Proposed 5-Stage Pipeline

A 5-stage pipeline is selected because it provides good instruction throughput without excessive hardware complexity.

IF → ID → EX → MEM → WB

Where:

- IF = Instruction Fetch

- ID = Instruction Decode

- EX = Execute

- MEM = Memory Access

- WB = Write Back

IF

Instruction Fetch

2.2 Pipeline Datapath

WB

Write Back

MEM

Memory Access

EX

Operation

ID

Instruction Decode

2.3 Application of Pipeline

Consider four instructions:

I1: LOAD R1, [Sensor]

I2: ADD  R2, R1, R3

I3: CMP  R2, R4

I4: STORE R2, [Memory]

Pipeline operation:

| Cycle | I1 | I2 | I3 | I4 |
| --- | --- | --- | --- | --- |
| 1 | IF |  |  |  |
| 2 | ID | IF |  |  |
| 3 | EX | ID | IF |  |
| 4 | MEM | EX | ID | IF |
| 5 | WB | MEM | EX | ID |
| 6 |  | WB | MEM | EX |
| 7 |  |  | WB | MEM |

# 2.4 Data Hazard – L4

A data hazard occurs when one instruction depends on the result of another instruction.

Example:

I1: ADD R1,R2,R3

I2: SUB R4,R1,R5

I2 requires the value produced by I1.

### Solution: Forwarding

Instead of waiting until I1 completes write-back, the result can be forwarded directly from the ALU pipeline stage to the next instruction.

### Analysis

Forwarding reduces pipeline stalls and improves throughput.

# 2.5 Load-Use Hazard

Example:

I1: LOAD R1,[1000]

I2: ADD R2,R1,R3

The data from the load instruction may not be available early enough.

### Solution

A one-cycle stall can be inserted.

IF → ID → STALL → EX → MEM → WB

This prevents incorrect data from being used.

# 2.6 Control Hazard

A control hazard occurs due to branch instructions.

Example:

BEQ R1,R2,Target

The processor may fetch the wrong next instruction before knowing whether the branch is taken.

### Solution: Branch Prediction

The processor predicts whether the branch will be taken.

Two approaches are:

- Static branch prediction

- Dynamic branch prediction

Dynamic prediction uses previous branch behavior to improve accuracy.

### Applied example

If a monitoring loop executes repeatedly:

while(sensor < threshold)

the branch is often predictable.

Therefore, branch prediction can reduce pipeline flushing.

# 2.7 Structural Hazard

A structural hazard occurs when two pipeline stages require the same hardware resource simultaneously.

Example:

Instruction Fetch → Memory

Data Access       → Same Memory

Both may require the same memory port.

### Solution

Use separate:

- Instruction cache

- Data cache

This is one reason for using an L1 instruction cache and L1 data cache.

2.8 Hazard Comparison

| Hazard | Cause | Example | Solution |
| --- | --- | --- | --- |
| Data | Instruction dependency | ADD followed by SUB | Forwarding |
| Load-use | Load result unavailable | LOAD followed by ADD | Stall + forwarding |
| Control | Branch decision | BEQ | Branch prediction |
| Structural | Resource conflict | Shared memory | Separate resources |

# 2.9 Superscalar Processing

A superscalar processor can issue multiple instructions in one clock cycle.

Example:

Cycle 1:

ADD + LOAD

### Advantage

Higher instruction throughput.

### Limitation

Requires more hardware and increases power consumption.

# 2.10 Out-of-Order Execution

In out-of-order execution, instructions are executed when their operands and resources are available rather than strictly following program order.

Example:

I1: LOAD R1

I2: ADD R2,R1,R3

I3: ADD R5,R6,R7

If I2 is waiting for memory, I3 can execute first

### Analysis – L4

This improves hardware utilization but requires:

- Reservation stations

- Register renaming

- Reorder buffer

- More control logic

Therefore, it increases complexity and power consumption.

# 2.11 Speculative Execution

Speculative execution allows the processor to execute instructions before it is certain that they are required.

Example:

if (temperature > threshold)

execute alarm routine

The processor predicts the branch and begins execution.

### Advantage

Improves performance.

### Disadvantage

Wrong predictions cause wasted work and increased energy consumption.

# 2.12 SMT – Simultaneous Multithreading

SMT allows multiple threads to use processor resources simultaneously.

### Suitability for industrial monitoring

SMT can improve utilization when several independent workloads exist:

Thread 1 → Sensor processing

Thread 2 → Communication

Thread 3 → Diagnostic monitoring

However, for a strongly real-time system, excessive sharing can make timing less predictable.

### Recommendation

Limited SMT or no SMT is recommended for the basic industrial processor unless multiple parallel workloads justify the extra complexity.

2.13 Recommendation

| Technique | Performance | Power | Complexity | Suitability |
| --- | --- | --- | --- | --- |
| Basic Pipeline | Medium | Low | Low | Excellent |
| Superscalar | High | Medium/High | High | Good |
| Out-of-Order | Very High | High | Very High | Moderate |
| Speculative Execution | High | Medium/High | High | Good |
| SMT | High for parallel workloads | Higher | High | Conditional |

# ASSESSMENT 3

# HIERARCHICAL MEMORY SYSTEM & PERFORMANCE EVALUATION

## CO4:

3.1 Proposed Memory Hierarchy

CPU

L1 Cache

SRAM - Fast

L2 Cache

SRAM

L3 Cache

Shared

Main Memory

DRAM

NAND Flash

Storage

# 3.2 L1 Cache

L1 is the smallest and fastest cache.

Proposed:

- L1 Instruction Cache = 32 KB

- L1 Data Cache = 32 KB

### Application

Frequently accessed sensor-processing instructions and data are kept in L1.

### Benefit

Very low latency.

# 3.3 L2 Cache

Proposed L2:

512 KB

L2 stores data not found in L1.

It is slower than L1 but larger.

# 3.4 L3 Cache

Proposed L3:

8 MB

L3 acts as the final cache before main memory.

It reduces expensive DRAM accesses.

# 3.5 Virtual Memory

Virtual memory allows programs to use virtual addresses.

The virtual address is divided into:

# 3.6 Cache Hit and Miss

### Cache Hit

Requested data is found in cache.

### Cache Miss

Requested data is not found and must be obtained from the next memory level.

# 3.7 AMAT Calculation

Average Memory Access Time:

Assume:

- L1 hit time = 1 ns

- L1 miss rate = 5%

- L2 access/miss penalty = 10 ns

Then:

### Analysis

If the miss rate increases to 10%:

# 3.8 Memory Technology Comparison

The assignment requires comparison of SRAM, DRAM, NAND Flash, MRAM and RRAM.

| Technology | Latency | Volatility | Density | Power | Suitable Application |
| --- | --- | --- | --- | --- | --- |
| SRAM | Very low | Volatile | Low | Higher per bit | L1/L2 cache |
| DRAM | Low/Medium | Volatile | High | Moderate | Main memory |
| NAND Flash | High | Non-volatile | Very high | Low standby | Storage |
| MRAM | Low | Non-volatile | High | Low | Embedded memory |
| RRAM | Potentially low | Non-volatile | High | Low | Future embedded/storage |

# 3.9 SRAM Analysis

SRAM is fast and does not require refresh.

Therefore:

Best application: processor cache.

Disadvantage:

- Larger area

- Higher cost per bit

- Lower density

# 3.10 DRAM Analysis

DRAM provides high density at lower cost per bit than SRAM.

It requires periodic refreshing.

Best application:

Main Memory

3.11 NAND Flash Analysis

NAND Flash is non-volatile and provides high storage density.

Best application:

Program Storage

Data Logging

Industrial Event Records

It is unsuitable as the primary CPU cache because its latency is much higher than SRAM.

# 3.12 MRAM Analysis

MRAM is non-volatile and can provide relatively fast access.

Potential benefits include:

- Non-volatility

- Good endurance

- Low standby power

- Fast access

It can be considered for embedded memory and persistent data.

3.13 RRAM Analysis

RRAM uses resistance states to store information.

Potential advantages:

- High density

- Non-volatility

- Low power potential

- Scalability

It is attractive for future embedded and storage applications.

# 3.14 Memory Hierarchy Recommendation

| Level | Recommended Technology | Reason |
| --- | --- | --- |
| L1 | SRAM | Lowest latency |
| L2 | SRAM | High-speed access |
| L3 | SRAM | Large cache with low latency |
| Main Memory | DRAM | High capacity |
| Persistent Storage | NAND Flash | Non-volatile, high density |
| Future Embedded Memory | MRAM/RRAM | Non-volatile and energy-efficient potential |

ASSESSMENT 4

RESULTS, COMPARISON & ENGINEERING RECOMMENDATION

4.1 Overall Architecture Comparison

| Feature | Conventional Design | Proposed Design |
| --- | --- | --- |
| Pipeline | Limited | 5-stage |
| Hazard Handling | Stalling | Forwarding + prediction |
| L1 Cache | Basic | Split I/D cache |
| L2 Cache | Basic | 512 KB |
| L3 Cache | Optional | 8 MB |
| Main Memory | DRAM | Low-power DRAM |
| Storage | Flash | NAND Flash |
| Virtual Memory | Basic | Paging + TLB |
| Execution | In-order | Controlled advanced techniques |
| Power Optimization | Limited | Clock/power management |
| Real-time Suitability | Medium | High |

# 4.2 Performance Analysis

Assume:

### Design A

CPI=1.5

### Design B

CPI=1.2

IC=1,000,000

and:

f=1GHzf=1GHz

### Design A

Design B

# 4.3 Bottleneck Analysis

The major possible bottlenecks are:

- Cache misses

- Branch misprediction

- Memory latency

- Pipeline hazards

- DRAM access

- Excessive power consumption

- I/O transfer delays

### Proposed solutions:

| Bottleneck | Solution |
| --- | --- |
| Cache misses | Larger L2/L3 + optimized locality |
| Data hazards | Forwarding |
| Load-use hazards | Controlled stalls |
| Branch hazards | Branch prediction |
| DRAM latency | Cache hierarchy |
| Power | Clock/power management |

# 4.4 Final Engineering Recommendation

The recommended architecture is:

### Recommendation

The processor should prioritize:

- Moderate clock frequency

- Low CPI

- Efficient pipeline

- Forwarding

- Branch prediction

- Large enough L2/L3 caches

- DRAM for main memory

- NAND Flash for persistent storage

- Low-power I/O

- Limited advanced execution features

A highly aggressive out-of-order and SMT design is not necessary for the primary application because it can increase power and complexity.

ASSESSMENT 5

REFLECTION, DESIGN DECISIONS, SDG RELEVANCE & LEARNING OUTCOMES

# 5.1 Design Decisions

The 5-stage pipeline was selected because it provides a reasonable balance between throughput and hardware complexity.

Forwarding was selected to reduce data hazards without introducing unnecessary stalls.

Branch prediction was selected to reduce control-hazard penalties.

A hierarchical cache was selected because the CPU is significantly faster than main memory. Keeping frequently used sensor data close to the processor reduces memory latency and energy consumption.

SRAM is used for caches because of its low latency, while DRAM is selected for main memory because it provides higher capacity at lower cost.

NAND Flash is selected for permanent storage because it is non-volatile and has high density.

# 5.2 SDG 7 – Affordable and Clean Energy

The design supports SDG 7 – Affordable and Clean Energy through energy-efficient processor operation.

Energy consumption can be reduced using:

- Lower unnecessary CPU activity

- Efficient cache usage

- Reduced memory accesses

- Pipeline optimization

- Clock gating

Reducing processor power consumption also reduces the energy required by industrial monitoring systems.

# 5.3 SDG 9 – Industry, Innovation and Infrastructure

The proposed processor directly supports SDG 9 – Industry, Innovation and Infrastructure.

Smart manufacturing requires:

- Real-time monitoring

- Automated decision-making

- Reliable infrastructure

- Efficient data processing

- Intelligent sensors

The proposed processor provides an architecture suitable for intelligent industrial systems.

# 5.4 SDG 12 – Responsible Consumption and Production

The architecture supports SDG 12 – Responsible Consumption and Production through efficient resource utilization.

Energy and hardware resources are used efficiently by:

- Reducing unnecessary computation

- Minimizing memory traffic

- Using suitable memory technologies

- Selecting appropriate cache sizes

- Reducing unnecessary hardware complexity

This helps reduce energy consumption and resource wastage.

# 5.5 Reflection

During the design process, the major challenge was balancing performance with energy consumption.

Increasing clock frequency can improve performance but generally increases power consumption. Similarly, advanced techniques such as out-of-order execution and SMT can improve utilization but require additional hardware and control logic.

The analysis demonstrates that the fastest architecture is not always the most suitable architecture for an industrial monitoring system.

The proposed design therefore emphasizes:

Possible future improvements include:

- AI/ML accelerator integration

- More advanced branch prediction

- MRAM/RRAM integration

- Adaptive voltage and frequency scaling

- Hardware accelerators for sensor processing

- More advanced power management